Supplementary data: Standard protocols and modified pregnant-woman protocol for both modalities, 18F-FDG PET and X-rays CT and the estimated reduction of fetal irradiation relatively to standard protocols.

| Imaging Modality | Standard Protocol | Pregnant-Woman Protocol | Estimated of the foetal irradiation reduction in % of Standard Protocol |
| --- | --- | --- | --- |
| 18F-FDG PET Injected radioactivity MBq/kg bw* | 2.5 to 5 | 1.5 | 40 to 70% |
| X-rays CT Intensity (mAs) Voltage (kV) Cut (mm) Pitch | 80 120 5 0.8 | 35 120 5 0.8 | - to 60% |

* bw : Body Weight; mAs : milli Ampere second; kV : kilovolt; mm : millimeter; Pitch is unitless