# Supplementary material for Grönlund E, et al. Dose painting of prostate cancer based on Gleason score correlations with apparent diffusion coefficients. Acta Oncol….

## Virtual voxelization of prostate volumes from biopsy data

The biopsy results for the learning set patients are used to construct virtual prostate volumes voxelized so that the observed relative frequency of G values per individual prostate are conserved. Spatial relationships were not considered since such data are not needed to determine the Gleason dependent dose-responses. The number of voxels assigned per prostate is given by

,	
where Vpat is the prostate volume in mm3 (determined with ultrasound) and  equals the chosen voxel size of 3 mm. Each biopsy sample were specified with the sample length ls and the observed cancer length ls,c in mm. Furthermore, the observed cancer per sample was assigned with a primary Gleason grade G1,s and a secondary Gleason grade G2,s, were the grading was performed according to the pathological guideline consensus used before the new consensus first presented in 2005 and updated in 2014 [1,2]. We calculate the Gleason score G per sample with a criteria to take into account the increased severity for example between (G1,s, G2,s) =(3,4) compared to (G1,s, G2,s) =(4,3) by applying half grade steps through

.	
This is in accordance with the new Gleason scoring consensus that grades (G1,s, G2,s) =(3,4) in a lower risk category than (G1,s, G2,s) =(4,3), consistent with observed follow-up data [2–4].

For each patient, the number of voxels  assigned with each observed G (given by Eq. 2) is set proportional to the total sampled length of each G as 
	,

where  is each patient’s number of voxels (given by Eq. 1),  the length of observed G for a sample,  the length of each sample, and  the number of samples per patient. The remaining portion of voxels corresponding to the sample lengths without any observed G are simply assigned with .

## Robustness test of dose painting prescriptions versus bias in voxelization data

To test how biopsy localization biases may affect the resulting dose painting prescriptions, we constructed a modified voxelization procedure. Instead of assuming that each biopsy sample per patient represented an unbiased subset of the prostate volume (as in Eq. 3), we reassigned voxels with  in proportion to the unsampled prostate regions per patient (given from chart data of the biopsies). For this modified voxelization procedure, we derived new G driven dose-response functions to generate new dose painting prescriptions. We compare the resulting DVHs generated from the two voxelization routes (in Fig. S1) for the two test set patients with the highest and lowest TCP increase (presented in Fig. 6 in the main paper).

Figure S1. (Left) DVHs for dose painting prescriptions of the test set patient with the highest TCP increase of 30% (shown in the main paper in Fig. 6 Left). (Right) DVHs for dose painting prescriptions of the test set patient with the lowest TCP increase of 4% (shown in the main paper in Fig 6 Right). For both graphs, the gray line represents the DVH for the original voxelization (given by Eq.3, denoted with “orig.”) and the black line the DVH for the modified voxelization (denoted “modif.”). Also shown in both graphs is the expected  for a homogeneous dose of  Gy and the expected for the dose painting prescriptions.

The dose painting prescriptions are very similar with each other (as shown in Fig. S1), where the modified voxelization with a higher frequency of  demonstrates a slightly higher differentiation in dose. This observation implies that the voxelization procedure is robust regarding optimization of dose painting prescriptions. However, the results demonstrate differences in estimated TCP, which is an effect from that fewer high G values are estimated with a worse voxel specific TCP prediction by the modified voxelization route.

## Derivation of Gleason score driven dose response functions from the learning data set

We will now derive voxel specific Gleason score driven dose-response functions set to be consistent with the 5-year freedom from biochemical recurrences  of the learning set. We parameterize the TCP for the patient population of the learning set (index “ls”) as

,	
where is chosen so  at  Gy equals  , and  set to 2.01 according to Dasu et al. [5]. The  functions we are deriving are (as in our earlier study [6]) parameterized with logistic sigmoidal function as

,	
where  is set constant as suggested by Vogelius et al. [7] since a variable  require access to observed failure frequencies at different doses, which we not had. Moreover,  is set as a function of  since Gleason scores are assumed to differentiate the dose-responses.

Now, assuming that the voxels comprising a prostate volume (CTVT) respond independently, the TCP for a patient is given by

,

where  is voxel specific doses and G the voxelized Gleason scores.

We use two requirements to derive the parameters for  (as described by Eq. 5). Firstly, we require for the homogeneous dose  that the assumed voxel specific dose-response relationship given as

,	
must reconstruct the observed 5-year . Secondly, we require for the homogeneous dose  that the dose-response derivative for the whole learning set population is reconstructed by the average of the dose-response derivatives for the learning set patients. The first requirement gives

where the average of  (given by Eq.  with ) for all  patients reconstructs the observed tumor control frequency . This determines b and m for the dose-response relationship (given by Eq. 7). However, since only 26 patients in our learning set had a , yielding too low statistics to determine , we determine this value with the reported 5-year  from Epstein et al. [4] where we for simplicity assume equal dose levels. Our second requirement gives

,	
where the dose-response derivative for the learning set population (acquired from the dose derivative of Eq. 4 with ) reconstructs the dose-response derivative averaged over all learning set patients (acquired from the dose derivative of Eq. 6 with ). Since  is given by a product (Eq. 6 with ) the product rule for derivatives (implemented into Eq. 9) yields

.

Furthermore, (with  in Eq. 5) the following applies

,

which substituted (in Eq. 10) yield  as

.

With  known (from Eq. 12),  can (with Eq. 5) be determined as

.	
Hence, all needed parameters for the Gleason driven dose-response functions (described by Eq. 5) are determined. These dose-response functions can now be used to optimize dose painting prescriptions for prostate cancer patients with the end-point of freedom from biochemical recurrences 5-years post-RT.

## References

[1] 	Epstein JI, Allsbrook Jr WC, Amin MB, et al. The 2005 International Society of Urological Pathology (ISUP) consensus conference on Gleason grading of prostatic carcinoma. Am. J. Surg. Pathol. 2005;29:1228–1242.

[2] 	Epstein JI, Egevad L, Amin MB, et al. The 2014 International Society of Urological Pathology (ISUP) Consensus Conference on Gleason Grading of Prostatic Carcinoma: definition of grading patterns and proposal for a new grading system. Am. J. Surg. Pathol. 2016;40:244–252.

[3] 	Magi-Galluzzi C, Montironi R, Epstein JI. Contemporary Gleason grading and novel Grade Groups in clinical practice: Curr. Opin. Urol. 2016;26:488–492.

[4] 	Epstein JI, Zelefsky MJ, Sjoberg DD, et al. A Contemporary Prostate Cancer Grading System: A Validated Alternative to the Gleason Score. Eur. Urol. 2016;69:428–435.

[5] 	Dasu A, Toma-Dasu I. Will intrafraction repair have negative consequences on extreme hypofractionation in prostate radiation therapy? Br. J. Radiol. 2015;88:20150588.

[6] 	Grönlund E, Johansson S, Montelius A, et al. Dose painting by numbers based on retrospectively determined recurrence probabilities. Radiother. Oncol. 2017;122:236–241.

[7] 	Vogelius IR, Håkansson K, Due AK, et al. Failure-probability driven dose painting. Med. Phys. 2013;40:081717.