# Supplemental A. The automated treatment planning process for VMAT

The VMAT-optimisation process was fully automated by an in-house developed Python-script both setting up initial optimisation objectives (Figure SA1) together with beam settings (Table SA1). The PTV-objectives at the bottom of figure SA1, were robust criteria that were applied for both CT-series in parallel. The remaining objectives were only applied for the original CT series.

Figur SA1 Initial objectives for the VMAT-optimisation.

Table SA2.  Plan and Beam settings

|  | cVMAT | ncVMAT |
| --- | --- | --- |
| VMAT type | Sliding window | Sliding window |
| Maximum MLC-speed | 2mm/degree | 2mm/degree |
| Maximum total MU | 750 | 750 |
| Maximum delivery time | 150 s | 150 s (transversal arc) plus 60 s (sagittal arc) |
| Arc 1 gantry angles | 178-280 degrees | 178-280 degrees |
| Arc 1 collimator angle | 5 degrees | 5 degrees |
| Arc 1 couch angle | 0 degrees | 0 degrees |
| Arc 2 gantry angles | - | 50-0 degrees |
| Arc 2 collimator angle | - | 85 degrees |
| Arc 2 couch angle | - | 90 degrees |

The weights of MaxEUD-objectives (Maximum mean dose restriction) were set according to clinical priorities (Table SA3).  The levels aimed for in these objectives were adapted to what was achievable during the optimisation process for each individual case. The heart was an exception were the objective was kept at 1 Gy throughout the whole optimisation.

Table SA3. Optimisation weights used autoplanning.

| OAR | Weight |
| --- | --- |
| Left Lung | 20 |
| Right lung | 10 |
| Contralateral breast | 10 |
| Thyroid | 2 |
| Trachea | 2 |
| Oesophagus | 2 |
| Humeral head | 2 |
| Heart* | 10 |

* Max EUD objective for the heart is fixed at 1 Gy throughout all optimisation rounds, and despite a low weight the punishment for heart doses dominated the OAR-optimisation function when the MHD exceeded approximately 2 Gy.

The automated optimisation process was separated into two phases:

- Phase 1. Individual adaption of the OAR-objectives. 5 optimisation rounds with 40-100 iterations starting with settings as in figure A.

  - After each optimisation round, read out mean dose to ipsilateral lung, contralateral lung, contralateral breast, oesophagus, trachea, thyroid gland and ipsilateral humeral head.

  - Then change the Max EUD value in Gy for these OARs to 70% of the achieved value after previous optimisation round.

  - The Max EUD objective for the heart was maintained at 1 Gy throughout all optimisation steps to provide an increased priority for this parameter for cases with difficult anatomy and mean heart doses pushing above 2 Gy.

- Phase 2. Optimise Target coverage, homogeneity and max doses through 5 optimisation rounds with 50 iterations each.

  - Initially, increase uniform dose weight for CTVs to 300

  - After each optimisation round D98 was read out for PTVp_Breast, PTVn and PTVn_IMN (noted PTVpc, PTVnc and PTVn_IMN in figure A)

  - When D98 deviated from 38.05 Gy and 36.05 Gy for the breast and lymph node PTVs, respectively, the weight (W) of the Min Dose for PTVs and corresponding CTVs, were modified according to the following formulas for breast and lymph nodes:

PTV-breast:  		W(new) = ((38.1 - Current D98) * 2.5 * W) + W

PTV-Lymphnodes:  	W(new) = ((36.1 - Current D98) * 2.5 * W) + W

  - Before each optimisation round, the regions with dose above 41Gy and 42 Gy were segmented as separate structures and assigned max-dose criteria at 41.5 Gy and 42 Gy, respectively, with weight 1000.

The full-automated planning process required 30-40 minutes for optimisation with no manual input required during the process.