Supplemental B. Robust planning and evaluation

Robust planning.

In the present study, the robust planning feature in the Raystation treatment planning system was used in a novel way. This Raystation feature allows an optimisation of two or more scenarios simultaneously. One alternative is to optimise with different set-up error scenarios, which have previously been explored to obtain robustness toward superficial breast deformations [1]. Alternatively, it can be used to optimise at two or more CT-series simultaneously as is demonstrated by simulating breast movement. This have previously been demonstrated by simulating a breast target movement of 15 mm in either anterior or lateral  directions [2]. In the present study, we generated an additional virtual CT-series as a copy of the original and in this series; a 15 mm PTV margin was added to the PTVp_Breast in the anterior and lateral direction (Figure SB1). The body contour of this CT-series was expanded 5 mm outside this expanded PTV. Density override was set to water for area between the original and the expanded body contour in a similar manner as the virtual bolus strategy used by others [3]. For pendulous breasts, an additional PTV-margin can be included in the inferior direction within one additional CT-series. This is important, as the optimisation process is slowed down proportionally to each additional scenario that is included.

Figure SB1. Transversal and sagittal view of the original and expanded PTVp_Breast and external body.

Optimisation criteria of minimum dose at 36.2 Gy and maximum dose at 42Gy was added for robust optimisation on PTVp_Breast in both CT-series to provide acceptable dose to the PTV in the worst case scenario of a 15 mm breast expansion.

In a clinical routine, the expanded body structure in the robust CT is used to verify with daily Cone Beam CT verification that breast deformations do not exceeded the 15 mm allowed for in the optimisation.

## Robust evaluation

To evaluate the success of the robust planning strategy all plan types were recalculated for all patient cases with two different scenarios:

- Recalculated in the generated CT-series with 15mm anterior and lateral breast expansion.

- Simulated set-up errors by moving the patient 5 mm both anteriorly and laterally.

Figure SB2 illustrates the dose effect for both scenarios as compared to the original for a representative patient.

Figure SB2. The left panel shows the original plans, the middle column shows doses after set-up displacements (5mm anterior and lateral) and the right column shows doses recalculated at the CT-series with the breast expanded 15 mm. From top to bottom the 3D-CRT-, cVMAT- and ncVMAT-plans are displayed.

## References

1.	Jensen, C.A., et al., Robustness of VMAT and 3DCRT plans toward setup errors in radiation therapy of locally advanced left-sided breast cancer with DIBH. Phys Med, 2018. 45: p. 12-18.

2.	Dunlop, A., et al., Evaluation of organ motion-based robust optimisation for VMAT planning for breast and internal mammary chain radiotherapy. Clin Transl Radiat Oncol, 2019. 16: p. 60-66.

3.	Rossi, M., et al., The robustness of VMAT radiotherapy for breast cancer with tissue deformations. Medical Dosimetry, 2021. 46(1): p. 86-93.