# Supplementary for An Experimental Setup for Proton Irradiation of a Murine Leg Model for Radiobiological Studies

# Supplemental Material

- A CAD-file outlining the water phantom dimension in detail is provided.

- Link to MC simulations: https://github.com/APTG/2022_DCPT_MICE

## Monte Carlo Simulations

Detailed Monte Carlo (MC) simulations were carried out using Geant4 and SHIELD-HIT12A. The water phantom was modeled using measured phantom dimensions (instead of nominal). MC simulations were calculated on a reference plan prepared in a clinical treatment planning system (Eclipse, Varian). The planning target volume was (laterally) 10 cm wide and 2.5 cm high with a longitudinal depth ranging from 6.5 to 10.0 cm depth in the phantom. A benchmarked beam model of beam-matched treatment rooms at DCPT was applied, taking the bending magnets and the emittance of the beam into account. Water at 20°C was simulated, i.e., density was reduced to 0.9982 g/cm³ for transport. Scoring was handled in two ways: 1) a fine-meshed (1 mm voxels) isocenter planes for visual inspection, as well as a depth-dose curve narrow enough to ensure lateral equilibrium along the scoring path. 2) The actual target positions of the irradiated mice legs were considered as 20 x 20 x 4 mm³ monolithic voxels which could be shifted in steps of 4 mm along 8 positions across the SOBP and its distal edge. This was used to estimate the average dose and radiation quality in a volume that covered both the mouse leg dimensions and the possible leg movements.

Quantities scored beyond relative dose were dose- and track-averaged LET in water (at density 1.000 g/cm³), for all particles, protons only, and primary protons only, following 25. Additionally, dose- and track-averaged Qeff 26 was reported for the setup. The MC simulations are summarized in Supplementary Table 1.

All simulation input files including the beam model and results are available at https://github.com/APTG/2022_DCPT_MICE.

- Supplementary Table 1: AAPM TG268 summary of Monte Carlo simulations parameters41–43.

- Supplementary Figure 1: Template guideline for functional RILD scoring.

- Supplementary Figure 2: Acute damage development and RILD development, all doses.