# Supplementary Tables: An Experimental Setup for Proton Irradiation of a Murine Leg Model for Radiobiological Studies

Supplementary

Supplementary Table 1: Monte Carlo Simulations

|  | Description |  | References |
| --- | --- | --- | --- |
| Item name | SHIELD-HIT12A | Geant4 |  |
| Version | v1.0.1-1-g9768828 | v11.0.3 | [39] |
| Validation | Depth-dose measurements, LET and Q validation against microdosimetry, Geant4, TOPAS, RayStation, PHITS | Depth-dose measurements, SHIELD-HIT12A, Eclipse |  |
| Timing | 100 jobs, 55 minutes CPU time each. | 35 jobs, 260 minutes CPU time each |  |
| Geometry | Modeled from actual measured dimensions. | Modeled from actual measured dimensions. |  |
| Source | Generated with Eclipse TPS, DCPT beam model for MC |  |  |
| Physics | SH12A defaults, but the lowest allowed energy loss per transportation step (DEMIN) is set to 0.025 MeV/nucleon, and the relative mean energy loss per transportation step (DELTAE) is set to 1 %. | Geant4 reference (“QGSP_BIC_HP_EMY”) physics list with 0.01 mm production cut for all particles and 0.2 mm maximum step length for charged particles | [40] |
| Scoring | Dose to water. LETd,t and Qeffd,t in water, with various secondary particles | Dose to water. LETd,t in water inc. all secondaries available online. |  |
| # histories | 4505e7 primary particles | 10e8 primary particles |  |

Table 1: AAPM TG268 summary of Monte Carlo simulations parameters41.

References

[39] Bassler N, Hansen DC, Lühr A, et al. SHIELD-HIT12A: a monte carlo particle transport program for ion therapy research. J Phys Conf Ser. 2014;:489.AQ5

[40] Bassler N, Grzanka L, Armin L, et al. SHIELD-HIT12A—User’s Guide. 2016-6-1. https://svn.nfit.au.dk/trac/shieldhit. 2017

[41] Sechopoulos I, Rogers DWO, Bazalova-Carter M, et al. RECORDS: improved reporting of montE CarlO RaDiation transport studies: Report of the AAPM research committee task group 268. Med Phys. 2018;45(1):e1–e5. doi: 10.1002/mp.12702.